MIPS assembler code can be indented, and left white space on a line is ignored.

All instructions must be on a single line. The # is means any text from the # to the end of a line is a comment and to be ignored. Strings are denoted by "'s marks around the string.

Assembly language programs are not compiled, they are assembled. So a program does not consist of statements and blocks of statements as in a HLL, but a number of instructions telling the computing machine how to execute.

These instructions are very basic, such as li $v0, 4 (put the value 4 in $v0). Because this is a complete change of perspective from a HLL, it is useful to explicitly make this point, and for the reader to take note of it.

In MIPS and most assembly languages in general, a "." before an text string means the token (string) that follows it is an assembler directive.

In this program the .text directive means the instructions that follow are part of a program text (i.e. the program), and to be assembled into a program and stored in the text region of memory.

The .data directive means that what follows is program data, and to be stored in the static data region of memory.

The .asciiz directive tells the assembler to interpret the data which follows it as an ASCII string.

In MIPS assembler any text string followed by a ":" is a label. A label is just a marker in the code that can be used in other statements.

The label main: assumes the program begins at the first line in the assembled program, the place to begin execution.

Any time a constant is included in an instruction, it is called an immediate value. The constant must be in the instruction itself, so the value 4 in the instruction li $a0,4 is an immediate value, but the string "Hello World" is a constant but not an immediate value.

Only instructions and labels can be defined in a text segment, and only data and labels can be defined in a data segment. You can have multiple data segments and multiple text segments in a program.

Operators are text strings like li, la, and syscall.

li means load the immediate value into the register (e.g. li $v0,4 means $v0 <- 4).

la means to load the address at the label into the register.

syscall is used to request a system service. System services provide access to the user console, disk drives, and any other external devices. The service to be executed is a number contained in the $v0 register. The services are: service 4 prints a string starting at the address of the memory contained in the $a0; and service 10 halts, or exit, the program.

Instructions are operators and their arguments. So li is an operator; li $v0,4 is an instruction.